Annex D

City of San Angelo

Safety Review Checklists

*Reviewed and Updated August 2016

Annex D

City of San Angelo

Safety Review Checklists

Purpose

One of the major goals of Risk Management is to reduce or eliminate workplace hazards and accidents. One of the most widely accepted ways to identify these workplace hazards is conduct Safety and Health Inspections. The only way that you can be certain of the actual situation is for each Department Director/Division Manager to look at their area’s operations on a periodic basis.

Conducting a Safety and Health Inspection by each Division Director/Division Manager is a must if they are to know where probable hazards exist and whether they are under control.

The Safety Checklists (Figure D1-D48 at the end of this section) are designed to assist each Department Director/Division Manager in this fact finding task. Conducting these inspections will give each Department Director/Division Manager some indication of where they should begin to make it a safer and more healthful worksite for all of their employees.

Each Department Director/Division Manager may wish to assign this task to a Supervisor who will then be responsible for conducting these self-inspections on a scheduled, periodic basis and then report the results to the Department Director/Division Manager.

Each Department/Division shall keep a record of the self-inspection reports in order to track items that need to be corrected. These records will also give each Department/Division a task checklist of identifying hazards so that each Department/Division can institute corrective control procedures.

These checklists are by no means all-inclusive. Each Department/Division may wish to add to them or delete portions that do not apply to them. Carefully consider each item to see if it really applies to the Department/Division’s operations.

Additionally, these self-inspection checklists will be used by Risk Management in conducting their yearly Safety Inspections.

1-1

Safety Checklists Contents

Office Safety                                                                         D1

Employee Posters                                                                D2

Record Keeping                                                                   D3

Safety Program & First Aid                                              D4

Fire Protection                                                                      D5

Personal Protective Equipment (PPE)                                 D6

General Work Environment                                                  D7

Walkways                                                                             D8

Floor and Wall Openings                                                      D9

Stairs and Stairways                                                             D10

Elevated Surfaces                                                                D11

Exiting or Egress                                                                  D12

Exit Doors                                                                             D13

Portable Ladders                                                                  D14

Hand Tools and Equipment                                                  D15

Portable (Power Operated) Tools and Equipment               D16

Abrasive Wheel Equipment-Grinders                                   D17

Power Actuated Tools                                                          D18

Machine Guarding                                                                D19

Lockout/Tag-Out Procedures                                               D20

Welding, Cutting and Brazing                                               D21

Compressors and Compressed Air                                      D22

Compressor Air Receivers                                                   D23

Compressed Gas Cylinders                                                 D24

Hoist and Auxiliary Equipment                                             D25

Industrial Trucks-Forklifts                                                     D26

Spraying Operations                                                            D27

Entering Confined Spaces                                                   D28

Environmental Controls                                                       D29

Flammable and Combustible Materials                               D30

Hazardous Chemical Exposure                                           D31

Hazardous Substance Communication                               D32

Electrical                                                                              D33

Noise                                                                                   D34

Fueling	                                                                       D35

Identification of Piping Systems                                           D36

Material Handling                                                                 D37

Transporting Employees and Materials                               D38

Control of Harmful Substances by Ventilation                     D39

Sanitizing Equipment and Clothing                                      D40

Tire Inflation                                                                         D41

Motor Vehicle Maintenance Shop                                        D42

Vehicle Operations                                                               D43

Vehicle Towing                                                                     D44

Knives                                                                                  D45

Mowing Operations                                                              D46

Swimming Pool Safety                                                         D47

Exposure Control for BBP & Infectious Materials                D48

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Office Safety

_____ Have personnel received an on-site safety orientation?

_____ Are employees trained on other subjects such as proper lifting, hazard recognition, hazard reporting, safety considerations of walking and sitting, ergonomics, etc?

_____ Are proper rules for lifting observed by employees when handling bulky office supplies and materials?

_____ Are hazardous parts of electric office machines and equipment effectively guarded?

_____ Are electrical cords in use?

_____ Are electrical cords of the approved heavy duty type?

_____ Is electrical equipment properly grounded to guard against the possibility of such and fire?

_____ Are circuits overloaded by plugging too many appliances into outlets?

_____ are aisles free of tripping hazards such as telephone cords, packages, wastepaper baskets, open file drawers, etc?

_____ Are only trained and qualified personnel permitted to operate office machines?

_____ Are chair mats in use?

_____ Do chair mats lay flat and not present a tripping hazard?

_____ Is suitable office equipment provided for stamping, sharpening and cutting operations?

_____ Are the use of makeshift tools, such as unprotected razor blades and pins, prohibited?

_____ Is the office furniture in good condition?

_____ Has broken equipment been tagged and removed from service?

_____ Are projections on bookcases, filing cabinets and desks removed or guarded?

_____ Are floors kept clean and free from dirt and debris?

_____ Are torn carpets and/or rough, splintered, uneven or other flooring defects repaired or has the hazard been suitably marked?

_____ Are stairs free of obstacles?

_____ Are stair steps in good condition?

_____ Are handrails available and in good condition?

_____ During inclement weather, are storm mats placed near entrances and are floors mopped frequently?

_____ Are extension cords, that are used for things like electric fans, disconnected from the power source at the end of each work day?

_____ Are electric fans adequately guarded?

Figure D1-1

Office Safety (continued)

_____ Are only non-flammable wastebaskets used?

_____ Are only approved cleaning materials/supplies used to clean office equipment?

_____ Are only small amounts of cleaning supplies/chemicals kept on hand?

_____ Are chairs, boxes or other objects used as substitutes for ladders?

_____ Are file cabinets and bookcases restrained from tipping by bolting several together or attaching them to the wall or floor?

_____ Are heavier objects placed in lower drawers/shelves and lighter items in upper ones?

_____ Do all employees open only one file cabinet drawer at a time?

_____ Do employees keep file cabinet drawers closed when not in use?

Figure D1-2

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Employee Posters

_____ Are the required workplace poster(s) displayed in a prominent location where all employees are likely to see them?

_____ Are emergency phone numbers posted where they can be readily found in case of emergency?

_____ In locations where employees may be exposed to any hazardous materials, has the appropriate information concerning employee access to medical and exposure records, Safety Data Sheets (SDS), etc been posted or otherwise made readily available to affected employees?

_____ Are signs concerning exiting the building, room capacities, floor loading, exposure to x-ray, microwave or other harmful radiation and/or bloodborne pathogens posted where appropriate?

Figure D2

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Record Keeping

_____ Are employee medical, safety, hazardous communication, bloodborne pathogens and/or other training records up to date?

_____ Are records maintained for the required period of time (some medical and other records must be maintained for up to 30 years)?

_____ Are operating permits and records up to date for such items as elevators, air pressure tanks, liquefied petroleum gas tanks, etc?

Figure D3

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Safety Program

_____ Does this Department/Division have an active safety program?

_____ Are records of safety meetings and lists of topics covered on file?

_____ Is there one person who is clearly responsible for the overall safety program in your Department/Division?

_____ Is there a safety committee or group of employees that meet regularly to discuss safety related items?

_____ Is there a working procedure/plan for handling in-house employee complaints regarding safety issues?

_____ Are employees kept advised of any safety related items/problems that are being solved/fixed and/or the time frame for successful completion?

First Aid

_____ Are emergency numbers posted for easy access?

_____ Are first aid kits easily available in each work area and are they periodically inspected and replenished as needed?

_____ Have first aid kit supplies and contents been approved by Risk management?

_____ Are eyewash stations available if required?

_____ Are body showers available if required?

Figure D4

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Fire Protection

_____ Is the fire alarm system certified?

_____ Is the fire alarm system tested at least annually?

_____ If there are interior stand pipes and valves, are they inspected regularly?

_____ Are all fire doors in good operating condition?

_____ Are fire doors unobstructed and protected against obstruction?

_____ Are sprinkler heads protected by metal guards, when exposed to potential physical damage?

_____ Is the proper clearance maintained below sprinkler heads?

_____ Are portable fire extinguishers provided in adequate number and type?

_____ Are fire extinguishers mounted in readily accessible locations?

_____ Are fire extinguishers checked/recharged regularly and noted on the inspection tags?

_____ Are employees periodically instructed in the use of fire extinguishers and fire protection procedures?

Figure D5

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Personal Protective Equipment (PPE)

_____ Are protective goggles or face shields provided and worn where there is any danger of flying particles or corrosive materials?

_____ Are approved safety glasses required to be worn at all times in areas where there is any danger of flying particles or corrosive materials?

_____ Are employees who need corrective lenses (glasses or contacts) required to wear ONLY approved safety glasses, protective goggles or other medically approved precautionary procedures when working in environments that may have harmful exposure hazards?

_____ Are protective gloves, aprons, shields or other means/methods provided in order to protect against cuts, corrosive liquids and/or chemicals?

_____ Are hard hats provided and worn where danger of falling objects exists?

_____ Is appropriate foot protection required and worn where there is potential risk of foot injuries from hot, corrosive, poisonous substances, falling objects, crushing or penetrating hazards?

_____ Are approved respirators provided for regular or emergency use where needed?

_____ Are all types of PPE maintained in a sanitary condition and ready for use?

_____ Are there eyewash stations and quick drench showers where needed?

_____ Is special equipment for electrical workers available, where needed?

_____ Where lunches are eaten on the premises, are they eaten in an area where there is no exposure to toxic materials or other health hazards?

_____ Is protection against the effects of occupational noise exposure provided when sound levels exceed OSHA permissible levels?

_____ Are adequate work procedures, protective clothing and equipment provided and used when cleaning up spilled toxic or otherwise hazardous materials or liquids?

Figure D6

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

General Work Environment

_____ Are all worksites kept clean and orderly?

_____ Are all work surfaces kept dry or are appropriate measures taken to assure that the surfaces are slip-resistant?

_____ Are all spilled materials or liquids cleaned up immediately?

Is combustible scrap, debris and waste stored safely and removed from the worksite promptly?

_____ Are accumulations of combustible dust routinely removed from elevated surfaces?

_____ Is combustible dust cleaned up with a vacuum system in order to prevent the dust going into suspension?

_____ Are covered metal waste cans used only for oily or paint soaked materials?

_____ Are paint spray booths, dip tanks, etc cleaned regularly?

_____ Are all toilets and washing facilities clean and sanitary?

_____ Are paint spray booths, dip tanks, etc cleaned regularly?

_____ Are all work areas adequately lighted?

_____ Are pits and floor openings covered or otherwise guarded?

Figure D7

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Walkways

_____ Are aisles and passageways kept clear?

_____ Are aisles and walkways marked as needed?

_____ Are wet surfaces covered with non-slip materials?

_____ Are holes in the floor, sidewalk or other walking surfaces repaired properly, covered or otherwise made safe?

Is there safe clearance for walking in aisles where motorized or mechanical handling equipment is operating?

_____ Are materials or equipment stored in such a way that sharp objects will not interfere with the walkway?

_____ Are spilled materials cleaned up immediately?

_____ Are changes of direction or in elevation readily identifiable?

_____ Are aisles or walkways that pass near moving or operating machinery, welding operations or similar operations arranged so that employees will not be subjected to a potential hazard?

_____ Is adequate headroom provided for the entire length of any aisle or walkway?

_____ Are standard guardrails provided wherever aisle or walkway surfaces are elevated more than 30 inches above any adjacent floor or the ground?

_____ Are bridges provided over conveyors and similar hazards?

Figure D8

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Floor and Wall Openings

_____ Are floor openings guarded by a cover, a guardrail or equivalent on all sides (except at an entrance to stairways or ladders)?

_____ Are toe-boards installed around the edges of permanent floor opening(s), where persons may pass below the openings?

_____ Is the glass in the windows, doors, glass walls and etc, which are subject to human impact, of sufficient thickness and type for the condition of use?

_____ Are grates or similar type covers over floor openings, such as floor drains, of such design that foot traffic or rolling equipment will not be affected by the grate spacing?

_____ Are unused portions of service pits, and pits not actually, in use either covered or protected by guardrails or equivalent?

Figure D9

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Stairs and Stairways

_____ Are standard stair rails or handrails on all stairways having four or more risers available?

_____ Are all stairways at least 22 inches wide?

_____ Do stairs/stairways have at least 6 foot 6 inches (6’ 6”) of clearance?

_____ Do stairs/stairways angle no more than 50 degrees and no less than 30 degrees?

_____ Are step risers on stairs uniform from top to bottom, with no riser spacing greater than 7 ½ inches?

_____ Are steps on the stairs/stairways designed, or provided with, a surface that renders them slip resistant?

_____ Are stairway handrails located between 30 and 34 inches above the leading edge of stair treads?

_____ Do stairway handrails have at least 1 ½ inches of clearance between the handrail and the wall or surface that they are mounted on?

_____ Are stairway handrails capable of withstanding a load of 200 pounds, applied in any direction?

_____ Where stairs/stairways exit directly into any area where vehicles may be operated, are adequate barriers and warnings provided to prevent employees from stepping into the path of traffic?

_____ Do stairway landings have a dimension measured, in the direction of travel, of at least equal to the width of the stairway?

_____ Is the vertical distance between stairway landings limited to 12 feet or less?

Figure D10

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Elevated Surfaces

_____ Are signs posted, when appropriate, showing the elevated surface load capacity?

_____ Are surfaces that are elevated more than 30 inches above the floor or ground provided with standard guardrails?

_____ Are all elevated surfaces, beneath which people of machinery could be exposed to falling objects, provided with standard 4 inch toe-boards?

_____ Is a permanent means of access and egress provided to elevated storage and work surfaces?

_____ Is required headroom provided where necessary?

_____ Is material on elevated surfaces piled, stacked or racked in a manner that prevents it from tipping, falling, collapsing, rolling or spreading?

_____ Are dock boards or bridge plates used when transferring material between docks and trucks or rail cars?

Figure D11

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Exiting or Egress

_____ Are all exits marked with an exit sign and illuminated by a reliable light source?

_____ Are the directions to exits, when not immediately apparent, marked with visible signs?

_____ Are doors, passageways or stairways, that are neither exits nor access to exits and which could not be mistaken for exits, appropriately marked “NOT AN EXIT”, “TO BASEMENT”’ “TO STOREROOM”, etc?

_____ Are exit signs provided with the word “EXIT” in lettering at least 5 inches high and the stroke of the lettering is at least ½ inch wide?

_____ Are all exit doors side hinged?

_____ Are all exit doors kept free of obstructions?

_____ Are there at least two (2) means of egress provided from elevated platforms, pits or rooms where the absence of a second exit would increase the risk of injury from hot, poisonous, corrosive, suffocating, flammable or explosive substances?

_____ Are there sufficient exits to permit prompt escape in case of an emergency?

_____ Are special precautions taken to protect employees during construction or repair operations?

_____ Is the number of exits from each floor of a building and the number of exits from the building itself, appropriate for the building occupancy load?

_____ Are exit stairways, which are required to be separated from other parts of the building, enclosed by at least  two (2) hour fire-resistant construction in buildings more than four (4) stories in height, and not less than  one (1) hour fire-resistant construction elsewhere?

_____ Where ramps are used as part of the required exiting from a building, is the ramp slope limited to one (1) foot vertical and 12 feet horizontal?

_____ Where exiting will be through frameless glass doors, glass exit doors, storm doors, etc, are the doors fully tempered and meet the safety requirements for human impact?

Figure D12

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Exit Doors

_____ Are doors which are required to serve as exits designed and constructed so that the way of travel is obvious and direct?

_____ Are windows, which could be mistaken for doors, made inaccessible by means of barriers or railings?

_____ Do exit doors open from the direction of exit travel without the use of a key or any special knowledge or effort when the building is occupied?

_____ Are revolving, sliding or overhead doors prohibited from serving as a required exit door?

_____ When panic hardware is installed on a required exit door, will it allow the door to open by applying a force of 15 pounds or less in the direction of the exit traffic?

_____ Are doors on cold storage rooms provided with an inside release mechanism which will release the latch and open the door even if it is padlocked or otherwise locked on the outside?

_____ Where exit doors open directly onto any street, alley or other area where vehicles may be operated, are adequate barriers and warnings provided to prevent employees stepping into the path of traffic?

_____ Are doors that swing in both directions and are located between rooms where there is frequent traffic, provided with viewing panels in each door?

Figure D13

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Portable Ladders

_____ Are all ladders maintained in good condition, joints between steps and slide rails tight, all hardware and fittings securely attached and moveable parts operating freely without binding or undue play?

_____ Are non-slip safety feet provided on each metal ladder?

_____ Are non-slip safety feet provided on each metal or rung ladder?

_____ Are ladder rungs and steps free of grease and/or oil?

_____ Is it prohibited to place a ladder in front of doors that open toward the ladder except when the door is blocked open, locked or guarded?

_____ Is it prohibited to place ladders on boxes, barrels or other unstable bases in order to obtain additional height?

____ Are employees instructed to face the ladder when ascending or descending a ladder?

_____ Are employees prohibited from using ladders that are broken, missing steps, rungs, cleats, have broken side rails or other faulty parts/equipment?

_____ Are employees instructed to not use the top step of ordinary stepladders as a step?

_____ When portable rung ladders are used to gain access to elevated platforms, roofs, etc, does the ladder always extend at least three (3) feet above the elevated surface?

_____ When portable rung or cleat-type ladders are used is the base placed so that slipping will not occur or has it been lashed/otherwise held in place?

_____ Are portable metal ladders legibly marked with signs reading “CAUTION-DO NOTUSE AROUND ELECTRICAL EQUIPMENT” OR EQIVALENT WORDING?

_____ Are employees prohibited from using ladders as guys, braces, skids, gin poles or for other than their intended purposes?

_____ Are employees instructed to only adjust an extension ladder while standing at the base (not while standing on the ladder or from a position above the ladder)?

_____ Are metal ladders inspected for damage on a daily basis?

_____ Are the rungs of a ladder uniformly spaced at twelve inches (12”)?

Figure D14

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Hand Tools and Equipment

_____ Are all tools and equipment used by employees at their workplace in good operating condition?

_____ Are hand tools such as chisels, punches, etc, which develop mushroom heads during use, reconditioned or replaced as necessary?

_____ Are broken or fractured handles on hammers, axes and similar equipment replaced promptly?

_____ Are worn out and bent wrenches replaced regularly?

_____ Are appropriate handles used on files, rasps and similar tools?

_____ Are employees made aware of the hazards caused by faulty or improperly used hand tools?

_____ Are appropriate safety glasses, face shields, etc, used while using hand tools or equipment which might produce flying materials or be subject to breakage?

_____ Are jacks checked periodically to ensure that they are in good operating condition?

_____ Are tool handles wedged tightly in the head of all tools?

_____ Are tool cutting edges kept sharp so that the tool will move smoothly without binding or skipping?

_____ Are tools stored in dry, secure locations where they won’t be tampered with?

_____ Is eye and/face protection used when driving hardened or tempered spuds or nails?

Figure D15

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Portable (Power Operated) Tools and Equipment

_____ Are grinders, saws and similar equipment provided with appropriate safety guards?

_____ Are power tools used with the correct shield, guard or attachment as recommended by the manufacturer?

_____ Are portable circular saws equipped with guards above and below the base shoe?

_____ Are circular saw guards checked to assure that they are not wedged up, thereby leaving the lower portion of the blade unguarded?

_____ Are rotating or moving parts of equipment guarded to prevent physical contact?

_____ Are all cord-connected, electrically-operated tools and equipment effectively grounded and/or are they the approved double-insulated type?

_____ Are effective guards in place over belts, pulleys, chains and/or sprockets on equipment such as concrete mixers, air conditioners, etc?

_____ Are portable fans provided with full guards or screens having openings of ½ inch or less?

_____ Is lifting/hoisting equipment available and used for lifting heavy equipment or objects and are the hoist ratings appropriate for the tasks?

_____ Are ground-fault circuit interrupters (GFCI) provided on all temporary electrical 15 and 20 amp circuits that are used during periods of construction?

_____ Are pneumatic and hydraulic hoses on power-operated tools checked regularly for deterioration or damage?

Figure D16

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Abrasive Wheel Equipment-Grinders

_____ Is the work rest used and kept adjusted to within 1/8 inch of the grinding wheel?

_____ Is the adjustable tongue on the top side of the grinder used and kept adjusted to within ¼ inch of the wheel?

_____ Do side guards cover the spindle, nut and flange and 75% of the wheel diameter?

_____ Are bench and pedestal grinders permanently mounted?

_____ Are goggles and/or face shields always worn when grinding?

_____ Is the maximum RPM rating of each abrasive wheel compatible with the RPM rating of the grinder motor?

_____ Are fixed or permanently mounted grinders connected to their electrical supply system with metallic conduit or other permanent wiring method?

_____ Does each grinder have an individual on and off control switch?

_____ Is each electrically operated grinder effectively grounded?

_____ before new abrasive wheels are mounted, are they visually inspected and ring tested?

_____ Are dust collectors and powered exhausts provided on grinders used in operations that produce large amounts of dust?

_____ Are splash guards mounted on grinders that use coolant in order to prevent the coolant from reaching employees?

_____ Is cleanliness maintained around grinders?

Figure D17

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Power Actuated Tools

_____ Are employees who operate power-actuated tools trained in their use and carry a valid operator’s card?

_____ Is each power-actuated tool stored in its own locked container when not being used?

_____ Is a sign at least seven (7) inches by ten (10) inches, with bold-faced type, reading “POWER ACTUATED TOOL IN USE” conspicuously posted when the tool is being used?

_____ Are power-actuated tools left unloaded until they are actually ready to be used?

_____ Do power-actuated tool operators have and use appropriate PPE such as hard hats, safety goggles, safety shoes/boots and ear protection?

Figure D18

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Machine Guarding

_____ Is there a training program to instruct employees on safe methods of machine operation?

_____ Is there adequate supervision to ensure that employees are following safe machine operating procedures?

_____ Is there a regular program of safety inspection of machinery and equipment?

_____ Is all machinery and equipment kept clean and properly maintained?

_____ Is sufficient clearance provided around and between machines to allow for safe operations, set up and servicing, material handling and waste removal?

_____ Is equipment and machinery securely placed and anchored, when necessary, to prevent tipping or other movement that could result in personal injury?

_____ Is there a power shut-off switch within reach of the operator’s position at each machine?

_____ Are the noncurrent-carry metal parts of electrically operated machines bonded and grounded?

_____ Are foot operated switches guarded or arranged to prevent accidental actuation by personnel or falling objects?

_____ Are manually operated valves and switches controlling the operation of equipment and machines clearly identified and readily accessible?

_____ Are all emergency stop buttons color coded red?

_____ Are pulleys and belts that are within seven (7) feet of the floor or working level properly guarded?

_____ Are all moving chains and gears properly guarded?

_____ Are splash guards mounted on machines that use coolant to prevent the coolant from reaching employees?

_____ Are methods provided to protect the operator and other employees in the machine area from hazards created at the point of operation, ingoing nip points, rotating parts, flying chips and sparks?

_____ Are machinery guards secure and so arranged that they do not offer a hazard in their use?

_____ When special hand tools are used for placing and removing material, do they protect the operator’s hands?

_____ Are revolving drums, barrels and containers required to be guarded by an enclosure that is interlocked with the drive mechanism, so that revolution cannot occur unless the guard enclosure is in place?

Figure D19-1

Machine Guarding (continued)

_____ Do arbors and mandrels have firm and secure bearings and are they free from play?

_____ Are provisions made to prevent machines from automatically starting when power is restored after a power failure or shutdown?

_____ Are machines constructed so as to be free from excessive vibration when the largest size tool is mounted and run at full speed?

_____If machinery is cleaned with compressed air, is the air pressure controlled and PPE or other safeguards utilized to protect operators and other workers from eye and/or body injury?

_____ Are fan blades protected with a guard that have openings no larger than ½ inch, when operating within seven (7) feet of the floor?

_____ Are saws used for ripping, equipped with anti-kickback devices and spreaders?

_____Are radial arm saws so arranged that the cutting head will gently return to the back of the table when released?

Figure D19-2

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Lockout/Tag-out Procedures

_____ Is all machinery and/or equipment capable of movement, required to be de-energizes or disengaged and blocked or locked-out during cleaning, servicing, adjusting or setting up operations, whenever required?

_____When disconnecting power for equipment does not also disconnect the electrical control circuit, are the appropriate electrical enclosures identified and are there ways provided to assure that the control circuit can also be disconnected and locked-out?

_____ Is the locking-out of control circuits in lieu of locking-out main power disconnects prohibited?

_____ Are all equipment control valve handles provided with a means of locking-out?

_____ Does the lockout/tag-out procedure require that stored energy (mechanical, hydraulic, air, etc) be released or blocked before equipment is locked-out for repairs?

_____ Are appropriate employees provided with individually keyed personal safety locks?

_____ Are employees required to keep personal control of their keys while they have safety locks in use?

_____ Is it required that the only employees exposed to the hazard, place or remove the safety locks?

_____ Is it required that employees check the safety of the lockout by attempting a start up after making sure that no one is exposed?

_____ Are employees instructed to always push the control circuit stop button prior to re-energizing the main power switch?

_____ Is there a means provided to identify any or all employees who are working on locked-out equipment by their locks or accompanying tags?

_____ Are a sufficient number of accident prevention signs or tags and safety padlocks provided for any reasonably foreseeable repair or emergency?

_____ When machine operations, configuration or size requires the operator to leave his/her control station to install tools or perform other operations, and that part of the machine could move if accidently activated, is such element required to be separately locked or tagged-out?

_____ In the event that equipment or lines cannot be shut down, locked-out and tagged, is a safe job procedure established and rigidly followed?

Figure D20

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Welding, Cutting and Brazing

_____ Are only authorized and trained personnel permitted to use welding, cutting or brazing equipment?

_____ Does each operator have a copy of the appropriate operating instructions and are they directed to follow them?

_____ Are compressed gas cylinders regularly examined for obvious signs of defects, deep rusting or leakage?

_____ Is care used in handling and storage of cylinders, safety valves, relief valves, etc, so as to prevent damage?

_____ Are precautions taken to prevent the mixing of air or oxygen with flammable gases, except at a burner or standard touch?

_____ Are only approved apparatus (torches, regulators, pressure reducing valves, acetylene generators, manifolds) used?

_____ Are cylinders kept away from elevators, stairs or gangways?

_____ Is it prohibited to use cylinders as rollers or supports?

_____ Are empty cylinders appropriately marked and their valves closed?

_____Are signs reading “DANGER-NO SMOKING, MATCHES OR OPEN LIGHTS” or the equivalent posted?

_____ Are cylinders, cylinder valves, couplings, regulators, hoses and apparatus kept free of oily and/or greasy substances?

_____ Is care taken not to drop or strike cylinders?

_____Unless secured on special trucks, are regulators removed and valve-protection caps put in place before moving cylinders?

_____ Are liquefied gases stored and shipped valve-end up with valve covers in place?

_____ Are provisions made to never crack/vent a fuel-gas cylinder valve near sources of ignition?

_____ Before a regulator is removed, is the valve closed and gas released from the regulator?

_____ Is red used to identify the acetylene (and other fuel-gas) hose, green for oxygen and black for inert gas and air hose?

_____ Are pressure-reducing regulators used only for the gas and pressures for which they are intended?

_____ Is open circuit (no-load) voltage of arc welding and cutting machines as low as possible and not in excess of the recommended limit?

_____ Under wet conditions, are automatic controls for reducing no-load voltage used?

_____ Are electrodes removed from the holders when not in use?

Figure D21-1

Welding, Cutting and Brazing (continued)

_____ Is it required that electric power to the welder be shut off when no one is in attendance?

_____ Is suitable fire extinguishing equipment available for immediate use?

_____ Is the welder forbidden to coil or loop welding electrode cable around his body?

_____ Are wet machines thoroughly dried and tested before being used?

_____ Are work and electrode lead cables frequently inspected for wear and damage, and replaced when needed?

_____ Do means for connecting cable lengths have adequate insulation?

_____ When the object being welded cannot be moved and fire hazards cannot be removed, are shields used to confine heat, sparks and slag?

_____ Are fire watchers assigned when welding or cutting is performed in locations where a serious fire might develop?

_____ Are combustible floors kept wet, covered by damp sand or protected by fire-resistant shields?

_____ When welding is done on metal walls, are precautions taken to protect combustibles on the other side?

_____ Before hot work is started, are used drums, barrels, tanks and other containers so thoroughly cleaned that no substances remain that could explode, ignite or produce toxic vapors?

_____ Is it required that eye protection, helmets, hand shields and goggles meet appropriate standards?

_____ Are employees exposed to the hazards created by welding, cutting or brazing operations protected with PPE or appropriate clothing?

_____ Is a check made for adequate ventilation in and where welding or cutting is performed?

___ When working in confined places, are environmental monitoring tests taken and means provided for quick removal of welders in case of an emergency?

Figure D21-2

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Compressors and Compressed Air

_____ Are compressors equipped with pressure relief valves and pressure gauges?

_____Are compressor air intakes installed and equipped so as to ensure that only clean uncontaminated air enters the compressor?

_____ Are air filters installed on the compressor intake?

_____ Are compressors operated and lubricated in accordance with the manufacturer’s recommendations?

_____ Are safety devices on compressed air systems checked frequently?

_____ Before any repair work is started on the pressure system of a compressor, is the pressure bled off and the system locked-out?

_____ Are signs posted to warn of the automatic starting features of the compressors?

_____ Is the belt drive system totally enclosed to provide protection for the front, back, top and sides?

_____ Is it strictly prohibited from using highly compressed air for cleaning purposes?

_____ If compressed air is used for cleaning off clothing off clothing, is the pressure reduced to less than 10psi?

_____ When using compressed air for cleaning, do employees wear protective chip guarding and PPE?

_____ Are safety chains or other suitable locking devices used at couplings of high pressure hose lines where a connection failure would create a hazard?

_____ Before compressed air is used to empty containers of liquid, is the safe working pressure of the container checked?

_____ When compressed air is used with abrasive blast cleaning equipment, is the operating valve a type that must be held open manually?

_____ When compressed air is used to inflate auto tires is a clip-on chuck and an inline regulator preset at the 40psi required?

_____ Is it prohibited to use compressed air to clean up or move combustible dust in the air and thereby create a potential fire or explosion hazard?

Figure D22

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Compressor Air Receivers

_____ Is every receiver equipped with a pressure gauge and one or more automatic, spring-loaded safety valves?

_____ Is the total relieving capacity of the safety valve capable of preventing pressure in the receiver from exceeding the maximum allowable working pressure of the receiver by more than ten (10) percent?

_____Is every air receiver provided with a drain pipe and valve at the lowest point for the removal of accumulated oil and water?

_____ Are compressed air receivers periodically drained of moisture and oil?

_____ Are all safety valves tested frequently and at regular intervals to determine whether they are in good operating condition?

_____ is the inlet of air receivers and piping systems kept free of accumulated oil?

Figure D23

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Compressed Gas Cylinders

_____ Are cylinders with a water weight capacity over 30 pounds equipped with means for connecting a valve protector device or with a collar or recess to recess to protect the valve?

_____ Are cylinders legibly marked to clearly identify the gas contained?

_____ Are compressed gas cylinders stored in areas which are protected from external heat sources such as flame impingement, intense radiant heat, electric arcs or high temperature lines?

_____ Are cylinders located or stored in areas such as flame they will not be damaged by passing or falling objects or subjected to tampering by unauthorized persons?

_____ Are cylinders stored or transported in a manner that will prevent them from creating a hazard by tipping, falling or rolling?

_____Are cylinders containing liquefied fuel gas stored or transported in a position so that the safety relief device is always in direct contact with the vapor space in the cylinder?

_____ Are valve protectors always placed on cylinders when the cylinders are not in use or connected for use?

_____Are all valves closed off before a cylinder is moved, when the cylinder is empty and at the completion of each job?

_____Are low pressure fuel-gas cylinders checked periodically for corrosion, general distortion, cracks or any other defect that might indicate a weakness or render it unfit for service?

_____ Does the periodic check of low pressure fuel-gas cylinders include a close inspection of the cylinder bottom?

Figure D24

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Hoist and Auxiliary Equipment

_____ Are only employees who have been trained in the proper use of hoists allowed to operate them?

_____ Is each overhead hoist with a limit device to stop the hook travel at its highest and lowest point of safe travel?

_____ Will each hoist automatically stop and hold any load up to 125 percent of its rated load, if its actuating force is removed?

_____ Is the rated load of each hoist legibly marked and visible to the operator?

_____ Are stops provided at the safe limits of travel for trolley hoists?

_____ Are the controls of hoists plainly marked to indicate the direction of travel or motion?

_____ Is each cage-controlled hoist equipped with an effective warning device?

_____ Are close-fitting guards or other suitable devices installed on hoists to assure hoist ropes will be maintained in the sheave groves?

_____ Are the hoist chains or ropes of sufficient length to handle the full range of movement of the application while still maintaining two (2) full wraps on the drum at all times?

_____ Are nip points or contact points between hoist ropes and sheaves which are permanently located within seven feet of the floor, ground or working platform, guarded?

_____ Is it prohibited to use chains or rope slings that are kinked or twisted?

_____ Is it prohibited to use the hoist rope or chain wrapped around the load as a substitute for a sling
_____ Is the operator instructed to avoid carrying loads over people?

Figure D25

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Industrial Trucks-Forklifts

_____ Are only trained personnel allowed to operate industrial trucks/forklifts?

_____ Is substantial overhead protective equipment provided on high lift rider equipment?

_____ Are the required lift truck operating rules posted and enforced?

_____ Is directional lighting provided on each industrial truck/forklift that operates in an area with low general lighting?

_____ Does each industrial truck/forklift have a warning horn, whistle, gong or other device which can be clearly heard above the normal noise in the areas where operated?

_____ Are the brakes on each industrial truck/forklift capable of bringing the vehicle to a complete and safe stop when fully loaded?

_____ Will the industrial truck/forklift parking brake effectively prevent the vehicle from moving when unattended?

_____ Are industrial trucks/forklifts operating in areas where flammable gases or vapors, or combustible dust or ignitable fibers may be present in the atmosphere, approved for such locations?

_____ Are motorized hand and/or hand/rider trucks so designed that the brakes are applied and power to the drive motor shuts off when the operator releases his/her grip on the devise that controls the travel?

_____ Are industrial trucks/forklifts with internal combustion engine, operated in buildings or enclosed areas, carefully checked to ensure that such operations do not cause harmful concentration of dangerous gases or fumes?

Figure D26

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Spraying Operations

_____ Is adequate ventilation assured before spray operations are started?

_____ Is mechanical ventilation provided when spraying operations?

_____ When mechanical ventilation is provided during spraying operations, is it so arranged that it will not circulate the contaminated air?

_____ Is the spray area free of hot surfaces?

_____ Is the spray area at least twenty (20) feet from flames, sparks, operating electrical motors and other sources of ignition?

_____ Are portable lamps used to illuminate spray areas suitable for use in a hazardous location?

_____ Is approved respiratory equipment provided and used when appropriate during spraying operation?

_____ Do solvents used for cleaning have a flash point that is 100degrees or more?

_____ Are fire control sprinkler heads kept clean?

_____ Are “NO SMOKING’ signs posted in spray areas, paint rooms, paint booths and storage areas?

_____ Is the spray area kept clean of combustible residue?

_____ Are spray booths constructed of metal, masonry or other substantial noncombustible material?

_____ Are spray booth floors and baffles noncombustible and easily cleaned?

_____ Is infrared drying apparatus kept out of the spray area during spraying operations?

_____ Is the spray booth completely ventilated before using the drying apparatus?

_____ Is the electric drying apparatus properly grounded?

_____ Are lighting fixtures for spray booths located outside of the booth and the interior lighted through sealed clear panels?

_____ Are the electric motors for exhaust fans placed outside booths or ducts?

_____ Are belts and pulleys inside the booth fully enclosed?

_____ Do ducts have access doors to allow cleaning?

_____ Do all drying spaces have adequate ventilation?

Figure D27

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Entering Confined Spaces

_____ Are confined spaces thoroughly emptied of any corrosive or hazardous substances, such as acids or caustics, before entry?

_____Are all lines to a confined space, containing inert, toxic, flammable or corrosive materials valves off and blanked or disconnected and separated before entry?

_____ Is it required that all impellers, agitators or other moving equipment inside confined spaces be locked-out if they present a hazard?

_____ Is either natural or mechanical ventilation provided prior to confined space entry?

_____ Are appropriate atmospheric tests performed to check for oxygen deficiency, toxic substances and explosive concentrations in the confined space before entry?

_____ Is adequate illumination provided for the work to be performed in the confined space?

_____ Is the atmosphere inside the confined space frequently tested or continually monitored while work is being performed?

_____ Is there an assigned safety standby employee outside of the confined space, when required, whose sole responsibility is to watch the work in progress, sound an alarm if necessary and to render assistance?

_____ Is the standby employee appropriately trained and equipped to handle an emergency?

_____ Is the standby employee, or other employees, prohibited from entering the confined space without lifelines and respiratory equipment if there is any questions as to the cause of an emergency?

_____ Is approved respiratory equipment required if the atmosphere inside the confined space cannot be made acceptable?

_____ Is all portable electrical equipment used inside confined spaces either grounded and insulated, or equipped with ground fault protections?

_____ Before gas welding or burning is started in a confined space, are hoses checked for leaks, are compressed gas bottles forbidden inside of the confined space, are torches lighted only outside of the confined area where it has been tested for an explosive atmosphere each time, before a lighted torch is to be taken into the confined space?

_____ If employees will be using oxygen-consuming equipment such as salamanders, torches, furnaces, etc, in a confined space, is sufficient air provided to assure combustion without reducing the oxygen concentration below 19.5 percent by volume?

_____ Whenever combustion-type equipment is used in a confined space, are provisions made to ensure that the exhaust gasses are vented outside of the enclosure?

Figure D28-1

Entering Confined Spaces (continued)

_____ Is each confined space checked for possible industrial waste which could contain toxic properties?

_____ If the confined space is below the ground and near areas where motor vehicles will be operating, is it possible for vehicle exhaust or carbon monoxide to enter the space?

Figure D28-2

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Environmental Controls

_____ Are all work areas properly illuminated?

_____ Are hazardous substances identified which may cause harm by inhalation, ingestions, skin absorption or contact?
_____ Are employees aware of the hazards involved with the various chemicals they may be exposed to in their work environment, such as ammonia, chlorine, epoxies, caustics, etc?

_____ Is employee exposure to chemicals in the workplace kept within acceptable levels?

_____Could a less harmful method or product be used?

_____ Is the work area’s ventilation system appropriate for the work being performed?

_____ Are spray painting operations done in spray rooms or booths that are equipped with an appropriate exhaust system?

_____ Is employee exposure to welding fumes controlled by ventilation, use of respirators, exposure time or other means?

_____ Are welders and other workers nearby provided with flash shields during welding operations?

_____If forklifts and/or other vehicles are used in buildings or other enclosed areas, are the carbon monoxide levels kept below maximum acceptable levels?

_____ Has there been a determination that noise levels in the facilities are within acceptable levels?

_____ Are steps being taken to use engineering controls to reduce excessive noise level?

_____ Are proper precautions being taken when handling asbestos and/or other fibrous materials?

_____ Are caution labels and signs used to warn of asbestos?

_____ Are wet methods used, when practicable, to prevent the emission of airborne asbestos fibers, silica dust and similar hazardous materials?

_____ Is vacuuming with appropriate equipment used whenever possible rather than blowing or sweeping?

_____Are grinders, saws and/or other machines that produce breathable dusts vented to an industrial collector or central exhaust system?

_____ Are all local exhaust ventilation systems designed and operating properly such as air flow and volume necessary for the application, and are ducts not plugged or belts slipping?

_____ Is PPE provided, used and maintained wherever required?

Figure D29-1

Environmental Controls (continued)

_____ Are there written standard operating procedures for the selection and use of respirators where needed?

_____ Are restrooms and washrooms kept clean and sanitary?

_____ Is all water provided for drinking, washing and cooking potable?

_____ Are all outlets for water not suitable for drinking clearly identified?

_____ Are employee physical capacities assessed before being assigned to jobs requiring heavy lifting?

_____ When/where heat is a problem, have all fixed work areas been provided with spot cooling or air conditioning?

_____ Are employees screened before assignment to areas of high heat to determine if their health condition might make them more susceptible to having an adverse reaction?

Are employees working on streets and roadways, where they are exposed to the hazards of traffic, required to wear bright colored (safety orange or green) warning vests?

_____ Are exhaust stacks and air intakes so located that contaminated air will not be re-circulated within a building or other enclosed area?

_____ Is equipment producing ultra-violet radiation properly shielded?

Figure D29-2

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Flammable and Combustible Materials

_____ Are combustible scrap, debris and waste materials (oily rags, etc) stored in covered metal receptacles and removed from the worksite promptly?

_____ Is proper storage practiced in order to minimize the risk of fire, including spontaneous combustion?

_____ Are approved containers and tanks used for the storage and handling of flammable and combustible?

_____ Are all connections on drums and combustible liquid piping, vapors and liquids tight?

_____ Are all flammable liquids kept in closed containers when not in use (parts cleaning tanks, pans, etc)?

_____ Are bulk drums of flammable liquids grounded and bonded to containers during dispensing?

_____ Do storage rooms for flammable and combustible liquids have explosion proof lights?

_____ Do storage rooms for flammable and combustible liquids have mechanical or gravity ventilation?

_____ Is liquid petroleum gas stored, handled and handled and used in accordance with safe practices and standards?

_____ Are “No Smoking” signs posted on liquid petroleum gas tanks?

_____ Are liquid petroleum storage tanks guarded in order to prevent damage from vehicles?

_____ Are all solvent wastes, and flammable liquids, kept in fire resistant, covered containers until they are removed from the worksite?

_____ Is vacuuming used, whenever possible, rather than blowing or sweeping when collecting combustible dust?

_____ Are firm separators placed between containers of combustibles or flammables, when they are stacked one upon another, in order to assure their support and stability?

_____ Are fuel gas cylinders and oxygen cylinders separated by distance, fire resistant barriers, etc, while in storage?

_____ Are fire extinguishers selected and provided for the types of materials in areas where they are to be used?

- Class A- Ordinary combustible material (wood, paper, cloth) fires

- Class B- Flammable liquid, gas or grease fires

- Class C- Energized-electrical equipment fires

- Class D- Combustible metals and hazardous metals

Figure D30-1

Flammable and Combustible Materials (continued)

_____ Are appropriate fire extinguishers mounted within 75 feet of outside areas containing flammable liquids, and within 10 feet of any inside storage area for such materials?

_____ Are all extinguishers free of obstructions or blockage?

_____ Are all extinguishers serviced, maintained and tagged at intervals not to exceed one (1) year?

_____ Are all extinguishers fully charged and in their designated places?

_____Where sprinkler systems are permanently installed, are the nozzle heads so directed or arranged that water will not be sprayed into operating electrical switch boards and equipment?

_____Are “NO SMOKING’ signs posted where appropriate in areas where flammable or combustible materials are used or stored?

_____Are safety cans used for dispensing flammable or combustible liquids at point of use?

_____ Are all spill of flammable or combustible liquids cleaned up promptly?

_____ Are storage tanks adequately vented in order to prevent the development of excessive vacuum or pressure as a result of filling, emptying or from atmosphere temperature changes?

_____ Are storage tanks equipped with emergency venting that will relieve internal pressure caused by fire exposure?

_____ Are ‘NO SMOKING’ rules enforced in areas involving storage and use of hazardous materials?

Figure D30-2

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Hazardous Chemical Exposure

_____ Are employees trained in the safe handling practices of hazardous chemicals such as acids, caustics, etc?

_____ Are employees aware of the potential hazards involving various chemicals stored or used in the workplace such as acids, bases, caustics, epoxies, phenols, etc?

_____ Is employee exposure to chemicals kept within acceptable levels?

_____ Are eye wash fountains and safety showers kept within acceptable levels?

_____ Are all containers, such as vats, storage tanks, etc, labeled as to their contents?

_____ Are all employees required to use appropriate PPE when handling chemicals (gloves, eye protection, respirators, etc)?

_____ Are flammable or toxic chemicals kept in closed containers when not in use?

_____ Are chemical piping systems clearly marked as to their content?

_____ Where corrosive liquids are frequently handled in open containers or drawn from storage vessels or pipe lines, is adequate means readily available for neutralizing or disposing of spills or overflows properly and safely?

_____ Have standard operating procedures been established and are they being followed when cleaning up chemical spills?

_____ Where needed for emergency use, are respirators stored in a convenient, clean and sanitary location?

_____ Are respirators intended for emergency use adequate for the various uses for which they may be needed?

_____ Are employees prohibited from eating in areas where hazardous chemicals are present?

_____ Is PPE provided, used and maintained whenever possible?

_____ Are there written standard operating procedures for the selection and use of respirators where needed?

_____ Are you familiar with the Threshold Limit values or Permissible Exposure Limits of airborne contaminants and physical agents use in your workplace (SDS)?

_____ Have control procedures been instituted for hazardous materials, where appropriate, such as respirators, ventilation systems, handling practices, etc?

_____ Whenever possible are hazardous substances handled in properly designed and exhausted booth or similar locations?

_____ Do you use general dilution or local exhaust ventilation systems to control dusts, vapors, gases, fumes smoke, solvents or midst which may be generated in your workplace?

Figure D31-1

Hazardous Chemical Exposure (continued)

_____ Is ventilation equipment provided for removal of contaminants from such operations as production grinding, buffing, spray painting and/or vapor degreasing, and is it operating properly?

_____ If internal combustion engines are used, is carbon monoxide monitored and exhausted outside?

_____ Is vacuuming used rather that blowing or sweeping dusts whenever possible?

_____ Are materials which give off toxic asphyxiant, suffocating or anesthetic fumes, stored in remote or isolated locations when not in use?

Figure D31-2

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Hazardous Substance Communication

_____ Is there a workplace list of hazardous substances in the workplace?

_____ Is there a written Hazardous Communication Program that deals with Safety Data Sheets (SDS), labeling and employee training?

_____ Is each container for a hazardous substance (vats, bottles, storage tanks, etc) labeled with product identity ad a hazardous warning (communication of the specific health and physical hazards)?

_____ Is there a SDS readily available for each hazardous substance used?

_____ Is there an employee training program for hazardous substances?

_____ Does the training program include:

- An explanation of what a SDS is and how to obtain and use one.

- SDS contents for each hazardous substance or class of substances.

- Explanation of “Right to Know”.

- Identification of where an employee can see the employer’s written hazard communication program and where hazardous substances are present in their work areas.

- The physical health hazards of substances in the work area and specific protective measures to be used.

- Details of the hazard communication program, including how to use the labeling system and SDS.

- Is a “Right to Know” poster displayed in the work area?

Figure D32

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Electrical

_____ Are all employees required to report, as soon as practical, any obvious hazards to life or property observed in connection with electrical equipment or lines?

_____ Are employees instructed to make preliminary inspections and/or appropriate tests to determine what conditions exist before starting work on electrical equipment or lines?

_____ When electrical equipment or lines are to be serviced, maintained or adjusted, are necessary switches opened, locked-out and tagged-out?

_____ Are portable electrical tools and equipment grounded or of the double insulated type?

_____ Are electrical appliances such as vacuum cleaners, polishers, vending machines, etc, grounded?

_____ Do extension cords being used have a grounding conductor?

_____ Are multiple plug adapters prohibited?

_____ Are ground fault circuit interrupters (GFCI) installed on each temporary 15 or 20 amp, 120 volt AC circuit at locations where construction, demolition, modifications, alterations or evacuations are being performed?

_____ Are all temporary circuits protected by suitable disconnecting switches or plug connectors at the junction with permanent wiring?

_____ Are there electrical installations in dust or vapor areas? Is so, do they meet the National Electrical Code (NEC) for hazardous locations?

_____ Is exposed wiring and cords with frayed or deteriorated insulation repaired or replaced promptly?

_____ Are flexible cords and cables free of splices or taps?

_____ Are clamps or other securing means provided on flexible cords or cables at plugs, receptacles, tools, equipment, etc, and is the cord jacket securely held in place?

_____ Are all cord, cable and raceway connections intact and secure?

_____ In wet or damp locations, are electrical tools and equipment appropriate for the use or location and/or otherwise protected?

_____ Is the location of electrical power lines and cables (overhead, underground, under-floor, other than side walls, etc) determined before digging, drilling or similar work is begun?

_____ Are metal measuring tapes, ropes, hand-lines or similar devices with metallic thread woven into the fabric prohibited where they could come in contact with energized parts of equipment or circuit conductors?

Figure D33-1

Electrical (continued)

_____ Is the use of metal ladders prohibited in areas where the ladder or the person using the ladder could come in contact with energized parts of equipment, fixtures or circuit conductors?

_____ Are all disconnecting switches and circuit breakers labeled so as to indicate their use or the equipment served?

_____ Do all interior wiring systems include provisions for grounding metal parts of electrical raceways, equipment and enclosures?

_____ Are all electrical raceways and enclosures securely fastened in place?

_____Are all energized parts of electrical circuits and equipment guarded against accidental contact by approved cabinets or enclosures?

_____ Is sufficient access and working space provided and maintained around electrical equipment so as to permit ready and safe operations and maintenance?

_____ Are all unused openings (including conduit knockouts) in electrical enclosures and fittings closed with appropriate covers, plugs or plates?

_____ Are electrical enclosures such as switches, receptacles, junction boxes, etc, provided with tight fitting covers or plates?

_____ Are disconnecting switches for electrical motors in excess of two (2) horsepower (hp) capable of opening the circuit when the motor is in a stalled condition, without exploding (switches must be hp rated equal to or in excess of the motor hp rating)?

_____ Is low voltage protection provided in the control device of motors driving machines or equipment which could cause probable injury from inadvertent startup?

_____ Is each motor disconnecting switch or circuit breaker located within sight of the motor control device?

_____ Is each motor located within sight of its controller or the controller disconnecting means capable of being locked in the open position or is a separate disconnecting means installed in the circuit within sight of the motor?

_____ Is the controller for each motor in excess of 2 hp equal to or in excess of the rating of the motor it serves?
_____ Are employees prohibited from working alone on energized lines or equipment over 600 volts?

Figure D33-2

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Noise

_____ Are there areas in the workplace where continuous noise levels exceed 90dBA?

_____ Is there an ongoing preventative health program to educate employees in safe levels of noise, exposures, and effects of noise on their health as well as the use of PPE?

_____ Have work areas where noise levels make voice communication between employees difficult been identified and posted?

_____ Are noise levels being measured using a sound level meter or an octave band analyser and records being kept?

_____ Have engineering controls been used to reduce excessive noise levels?

_____ Where engineering controls are determined to not be feasible, are administrative controls (i.e. worker rotation, etc) being used to minimize individual employee exposure to noise?

_____ Is approved hearing protective equipment available to every employee working in noisy areas?

_____ If you use ear protectors, are employees properly fitted and instructed in their use?

_____ Are employees in high noise areas given periodic audiometric testing to ensure that they are using an effective hearing protection system?

Figure D34

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Fueling

_____ Is it prohibited to fuel an internal combustion engine with a flammable liquid while the engine is running and are appropriate signs posted?

_____ Are fueling operations done in such a manner that likelihood of spillage will be minimal?

_____ When spillage occurs during fueling operations, is the spilled fuel washed away completely, evaporated or other measures taken to control vapors before restarting the engine?

_____ Are fuel tank caps replaced and secured before starting the engine?

_____ In fueling operations, is there always a metal contact between the container and the fuel tank?

_____ Are fueling hoses of a type designed to handle the specific type of fuel?

_____ Is it prohibited to handle or transfer gasoline in open containers?
_____ Are open lights, open flames, sparking or acing equipment prohibited near fueling or transfer of fuel operations?

_____ Is smoking prohibited in the vicinity of fueling operations and are appropriate signs posted?

_____ Are fueling operations prohibited in buildings or other enclosed areas that are not specifically ventilated for this purpose?

_____ Where fueling or transfer of fuel is done through a gravity flow system, are the nozzles of the self-closing type?

Figure D35

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Identification of Piping Systems

_____ When non-potable water is piped through a facility, are outlets and/or taps posted in order to alert employees that it is unsafe and not to be used for drinking, washing or other personal use?

_____When hazardous substances are transported through above ground piping, is each pipeline identified at points where confusion could introduce hazards to employees?

_____ When pipelines are identified by color painting, are all visible parts of the line so identified?

_____ When pipelines are identified by color painted bands or tapes, are the bands or tapes located at reasonable intervals and at each outlet, valve or valve connection?

_____ When pipelines are identified by color, is the color code posted at all locations where confusion could introduce hazards to employees?

_____ When the contents of pipelines are identified by name or name abbreviation, is the information readily visible on the pipe near each valve or outlet?

_____ When pipelines carrying hazardous substances are identified by tags, are the tags constructed of durable materials, the message carried clearly and permanently distinguishable and are tags installed at each valve or outlet?

_____ When pipelines are heated by electricity, steam or other external source, are suitable warning signs or tags placed at unions, valves and/or other serviceable parts of the system?

Figure D36

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Material Handling

_____ Is there safe clearance for equipment through aisles and doorways?

_____ Are aisleways designated, permanently marked and kept clear in order to allow unhindered passage?

_____ Are motorized vehicles and mechanized equipment inspected daily prior to use?

_____ Are vehicles shut off and the brakes set prior to loading or unloading?

_____ Are containers of combustibles or flammables, when stacked while being moved, always separated by dunnage sufficient to provide stability?

_____ Are dock boards (bridge plates) used when loading or unloading operations are taking place between vehicles and docks?

_____ Are trucks and trailers secured from movement during loading and/or unloading operations?

_____ Are dock plates and loading ramps constructed and maintained with sufficient strength to support imposed loading?

_____ Are hand-trucks maintained in safe operating condition?

_____ Are chutes equipped with sideboards of sufficient height to prevent the materials being handled from falling off?

_____ Are chutes and gravity roller sections firmly placed or secured so as to prevent displacement?

_____ At the delivery end of the rollers or chutes, are provisions made to brake the movement of the handled materials?

_____ Are pallets usually inspected before being loaded or moved?

_____ Are hooks with safety latches, or other arrangements, used when hoisting materials so that slings or load attachments won’t accidentally slip off the hoist hooks?

_____ Are securing chains, ropes, chocks or slings adequate for the job to be performed?

_____When hoisting material or equipment, are provisions made to assure no one will be passing under the suspended load?

_____ Are SDS available to employees handling hazardous substances?

Figure D37

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Transporting Employees and Materials

_____ Do all employees who operate vehicles on public roads have a valid operator’s license?

_____ When 15 or more employees are regularly transported in a van, bus or truck, is the operator’s license appropriate for the class of vehicle being driven?

_____Is each van, bus or truck that is regularly used to transport employees, equipped with an adequate number of seats and seat belts?

_____ When employees are transported by a truck, are provisions provided to prevent their falling from the vehicle?

_____ Are vehicles used to transport employees equipped with headlights, brakes, horn, mirrors, windshield and turn signals that are in good working order?

_____ Are transport vehicles provided with handrails, steps, stirrups, or similar devices, that are so placed or arranged that employees can safely mount and/or dismount?

_____ Are employee transport vehicles equipped, at all times, with at least two (2) reflective type flares?

_____ Is a fully charged fire extinguisher, in good condition, with at least a BC rating, maintained in each employee transport vehicle?

_____ When cutting tools, or tools with sharp edges, are carried in passenger compartments of employee transport vehicles, are they placed in closed boxes or containers which are secured in place?

_____ Are employees prohibited from riding on top of any load which can shift, topple, or otherwise become unstable?

Figure D38

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Control of Harmful Substances by Ventilation

_____ Is the volume and velocity of air in each exhaust system sufficient to gather the dusts, fumes, mists, vapors or gasses to be controlled and to convey them to a suitable point of disposal?

_____ Are exhaust inlets, ducts and plenums designed, constructed and supported so as to prevent collapse of failure of any part of the system?

_____ Are clean-out ports or doors provided at intervals not to exceed twelve (12) feet in all horizontal runs of exhaust ducts?

_____ Where two (2) or more different types of operations are being controlled through the same exhaust system, will the combination of substances being controlled constitute a fire, explosion or chemical reaction hazard in the duct?

_____ Is adequate makeup air provided to areas where exhaust systems are operating?

_____ Is the source point for makeup air located so that only clean, fresh air, which is free of contaminates, will enter the work environment?

_____ Where two (2) or more ventilation systems are serving a work area, is their operation such that one will not offset the functions of the other?

Figure D39

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Sanitizing Equipment and Clothing

_____ Is PPE/clothing that employees are required to wear/use capable of being cleaned easily and disinfected?

_____ Are employees prohibited from interchanging PPE/clothing, unless it has been properly cleaned?

_____ Are machines and equipment which process, handle or apply materials that could be injurious to employees, cleaned and/or decontaminated before being overhauled or placed in storage?

_____ Are employees prohibited from smoking or eating in any area where contaminates that could be injurious to employees if ingested are present?

_____ When employees are required to change from street clothing into protective clothing, is a clean change room with separate storage facility for street and protective clothing provided?

_____ Are employees required to shower and wash their hair as soon as possible after a known contact has occurred with a hazardous substance?

Figure D40

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Tire Inflation

_____ Where tires are mounted and/or inflated on drop center wheels, is a safe practice procedure posted and enforced?

_____ Where tires are mounted and/or inflated on wheels with split rims and/or retainer rings, is a safe practice procedure posted and enforced?

_____ Does each tire inflation hose have a clip-on chuck with at least 24 inches of hose between the chuck and in-line hand valve and gauge?

_____ Does the tire inflation control valve automatically shut-off the air flow when the valve is released?

_____ Is a tire restraining device such as a cage, rack or other effective used while inflating tires mounted on split rims or rims using retainer rings?

_____ Are employees strictly forbidden from taking a position directly over in front of a tire while it’s being inflated?

Figure D41

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Motor Vehicle Maintenance Shop

_____ Are adequate safety standards prescribed and observed by all shop personnel?

_____ Is painting, welding and battery work carried on in separate parts of the shop?

_____ Are shops adequately lighted?

_____ Are explosion proof fixtures and equipment in use in the vehicle paint shop?

_____ Is electrically powered machinery effectively guarded?

_____ Is flame producing equipment restricted to the welding shop?

_____ Are fire extinguishers conveniently located throughout the maintenance shop?

_____ Are un-purged fuel tanks permitted in the vehicle maintenance shops?

_____ Are “hot” operations performed on un-purged vehicles?

_____ Are special exhaust systems provided in battery rooms, painting booths and for confined welding jobs?

_____ Is flexible tubing available and utilized to carry carbon monoxide fumes directly outside the shop?

_____ Are shop floors maintained clean and free of oil, grease, gasoline, water or other hazardous and slippery materials?

_____ Are grease traps cleaned out on a regular basis?

_____ Is PPE available, worn properly and used correctly?

_____ Is shop machinery such as lathes, abrasive wheels and portable electric tools adequately guarded?

_____ Are vehicle lifts equipped with a safety device to prevent unintentional and/or accidental lowering?

_____ Are air-oil operated hydraulic lifts equipped with a lock which prevents raising by air if the oil supply is low?

_____ Are makeshift devices used for chassis and axle supports?

_____ Are guard cages used when inflating tires?

_____ After a vehicle has been raised by a jack, is it securely blocked to prevent its falling?

_____ Are hand tools and power tools maintained in good condition?

_____ Are appropriate traffic signs posted at entrances and exits of the shop?

_____ Do vehicles entering or leaving the shops signal with their horns?

_____ Is safe use of compressed air in effect?

Figure D42

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Vehicle Operations

_____ Are all drivers aware that they are responsible for the safe loading of their vehicles?

_____ Is cargo loaded in such a way that it will not shift or fall from the vehicle?

_____ Are red flags or lights used when loads project more than four feet beyond the tailgate or bed edge of a vehicle?

_____ Do drivers permit the truck cab to be overloaded with extra passengers?

_____ Are personnel allowed to ride in cargo spaces when there is danger of falling, rolling or shifting loads?

_____ Do drivers observe all traffic laws and signs?

_____ Do drivers operate their vehicles at speeds which will give them complete control under all circumstances?

_____ Do drivers give appropriate signals well in advance before making a turn?

_____ When parking, do drivers turn off the ignition and apply the parking brake?
_____ Does the driver make a positive check for clearance to the rear before backing his/her vehicle?

_____ Do drivers use spotters, when available, for backing?

_____ Are safe following distances always maintained behind the vehicle ahead in accordance with the speed and traffic conditions?

_____ Are windshields kept clean at all times in order to allow for good visibility?

_____ Are fogged, frozen or snow covered windshields and windows cleared and kept clean while operating the vehicle?

_____ Are exhaust systems inspected regularly to prevent carbon monoxide leakage?

_____ Does the driver always yield the right of way to pedestrians even though traffic signals may be in his/her favor?

_____ When driving in fog, do drivers use the low headlight beam rather than the high beam?

_____ Are drivers aware of the distance required to stop at various speeds and road conditions?

_____ Is the ignition always turned off before refueling the vehicle?

_____ Have all drivers taken the Defensive Driving Course in the last three (3) years?

Figure D43

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Vehicle Towing

_____ Are proper towing vehicles used to tow a disabled vehicle?

_____ Is a commercial towing company used when vehicle maintenance does not have a proper towing vehicle?

_____ Is a tow bar used when appropriate?

_____ Are chains used to tow vehicles?

_____ Do tow vehicle drivers use appropriate lights and signs when towing?

_____ Do tow vehicle drivers lower their speed to accommodate for road and traffic conditions?

Figure D44

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Knives

_____ Are personnel who use knives instructed in their safe use?

_____ Are knives equipped with hand shields or sure grip handles to keep user’s hands from slipping down on the blades?

_____ Do workers always cut away from their bodies when using knives?

_____ When not in use, are knives kept in racks with their edges guarded or stored in safe places/sheaths?

Figure D45

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Mowing Operations

_____ Is roll bar protection installed on/used on riding mowers and tractors?

_____ Are seatbelts provided?

_____ Do employees use the seatbelts that are provided?

_____ Are slow moving vehicles provided with “Slow Moving Vehicle” emblems?

_____ Are employees trained on proper steering, tip-over control, slope identification and PPE that is to be used when operating riding movers/tractors?
_____ Do operators leave the mowers/tractors running and unattended?

_____ Are hardhats, ear protection and eye protection used when operating riding mowers/tractors?

_____ Are safety orange or green vests used by mower/tractor operators?

_____ Are flags and/or flashers provided on all mowers/tractors?

_____ Are screens, chains or other material used as guards on mowers/tractors?

_____ Do operators “police the area” to be mowed and pick up foreign objects and identify potential trouble spots?

_____ Are operators instructed to move slowly in tall weeds?

_____ Are operators instructed to avoid sharp turns?

_____ Do operators avoid mowing wet slopes?

_____ Are operators instructed on how to mow on slopes?

_____ Before starting any activity off of the mower/tractor, does the operator insure that the engine is off and the blades stopped?

_____ Are operators instructed not to clear clogged blades with their hands?

_____ Are operators instructed on proper furling procedures?

_____ Are operators instructed on traffic safety while on public streets?

Figure D46

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Swimming Pool Safety

_____ Is the pool enclosed by a fence or other barrier

_____ Is depth conspicuously marked on the sides of the pool?

_____ If a diving board is used, is the depth in the diving area adequate for the height of the diving board?

_____ Is the pool adequately lighted?

_____ Is the water tested for contaminations?

_____ Is the water continuously circulated, filtered and treated?

_____ Are life rings and shepherd’s crooks provided?

_____ Are trained lifeguards always on duty when the pool is open?

_____ Does the lifeguard on duty stay by the pool?
_____ Is an attendant always on duty when the pool is open?
_____ Are unattended children allowed to use the pool?

_____ Are pool rules posted?

_____ Is the surface around the pool of a non-slip material?

_____ Is the pool and pool area free of trash, broken bottles or other hazardous materials?
_____ Are drinks dispensed only in paper or plastic cups/containers?

Figure D47

City of San Angelo

Safety Inspection Checklist

Department/Division: ____________________________    Date: _________________

Inspector: __________________________       Location: ________________________

Person Contacted: ______________________________________________________

Exposure Control for BBP and Other Infectious Materials

_____ Is a copy of the Exposure Control Plan available to employees?

_____ Are “universal (standard) precautions” always used by employees?

_____ Is PPE provided to employees?

_____ Do employees use their PPE?

_____ Are BBP and other infectious materials cleaned up properly when there is a spill?

_____ Are clean-up kits available?

_____ Are the HBV vaccination and biohazard signs properly posted?

_____ Are biohazard disposal receptacles available and used?

_____ Are biohazards disposed of properly?

_____ Is the Exposure Control Officer (Risk Manager) identified in writing?

_____ Does the Exposure Control Officer (Risk Manager) investigate all suspected exposures and provide a written report?

_____ Is the Training/Development Specialist identified in writing?

_____ Are training topics appropriate for the department?

_____ Are training records maintained?

_____ Are medical records maintained and kept in a locked file cabinet?

_____ Are Job Description lists maintained for each task in the department?

_____ Are employees properly trained when they move to a new job or task?

Figure D48