Vehicle/Trailer Inspection Test

Name: __________________________________         Date: ________________

Department: _____________________________

- When beginning an inspection of a trailer, where on the trailer will you start?

- The left front corner

- The left rear corner

- The right front corner

- The right rear corner

- When inspecting the trailer hitch system, what items are you checking?

- Coupler size, locking lever, latch, pin

- Gross trailer weight, jack stand, chock blocks

- Safety chains, electrical plugs

- All of the above

- When inspecting a tire, how do you determine that the tire tread depth is safe?

- By the tread wear indicator

- Take it to vehicle maintenance

- The head of a coin

- None of the above

- How many people are required to complete the inspection when checking the lights?

- One

- Two

- Three

- Four

- You need to transport a skid that weighs 5,705 lbs, what size of trailer do you need?

- 5,000 lb GTW

- 10,000 lb GTW

- 15,000 lb GTW

- 20,000 lb GTW

- Based on question 5, what is the minimum size ball hitch that you can use?

- 1 7/8 inch works for up to 2,000 lbs

- 2 inch works for up to 8,000 lbs

- 2 5/16 works for up to 30,000

- None of the above

- When inspecting a vehicle, which fluid is to be inspected daily?

- Brakes

- Coolant

- Engine Oil

- Transmission

- You can use a pipe (cheater bar) when tightening a chain with a pull-over-center binder?

- True

- False

- When using a trailer, if you are transporting a pallet of cargo that measures 4 ft wide x 4 ft long x 4 ft high how many straps should you use  to safely secure the load?

- One

- Two

- Three

- None

- Use _______ tie-downs if the cargo is longer than 5 feet, but shorter than 10 feet?

- One

- Two

- Three

- One for every 2 feet

- Chains come in 5 different grades, which type(s) are classified for transporting a load?

- Grade 30 & 43

- Grade 70

- Grade 70, 80 & 100

- Grade 80 & 100

- When transporting a riding mower on a trailer, how many straps should be used to safely secure it?

- One

- Two

- Three

- Four

- When securing a load to a trailer, the strap hooks are anchored to_______?

- The bump rail

- The frame

- The rear gate

- Where it is convenient

- According to the North American Cargo Securement Standard, every load that must be secured needs to withstand ______% (of cargo weight) forward force, ______% rearward force, ______% sideway force and ______% upward force?

- 50, 50, 20, 20

- 80, 50, 50 20

- 80 20,20 50

- 20, 80, 50, 20

- When conducting an inspection and you find a defect you should check the appropriate box, notate it in the section on the bottom of the form and notify your supervisor?

- True

- False

For questions 16-20, complete the attached inspection sheets accordingly:

- Coolant level is low

- One of the headlights is not working

- Brake lights are not working

- On the Trailer Inspection sheet, circle the number where combined weight rating is entered

- On the Trailer Inspection sheet, circle which hitch ball will accommodate 5,000lbs